Joint Crediting Mechanism Approved Methodology ID_AM026

“Introduction of CNG-Diesel Hybrid Equipment to Public Buses”

- Title of the methodology

Introduction of CNG-Diesel Hybrid Equipment to Public Buses, Version 01.0

- Terms and definitions

| Terms | Definitions |
| --- | --- |
| CNG-diesel hybrid equipment | Equipment (i.e. CNG fuel tank, fuel pressure regulator, fuel injector) which enables diesel engines to combust diesel fuel and CNG at the same time |

- Summary of the methodology

| - Items | - Summary |
| --- | --- |
| - GHG emission reduction measures | - Introduction of CNG-diesel hybrid equipment to public buses with diesel engines enables to replace diesel fuel with CNG, and also enables to improve fuel efficiency of buses, which lead to GHG emission reductions. |
| - Calculation of reference emissions | Reference emissions are emissions from buses without CNG-diesel hybrid equipment which consume diesel fuel only. They are calculated with CNG and diesel fuel consumption by project buses, net calorific value of CNG and diesel fuel, CO2 emission factor of diesel fuel, and fuel efficiency of project bus and reference bus. |
| - Calculation of project emissions | - Project emissions are emissions from buses with CNG-diesel hybrid equipment which consume both CNG and diesel fuel. They are calculated with CNG and diesel fuel consumption by project buses, net calorific value of CNG and diesel fuel, and CO2 emission factor of CNG and diesel fuel. - Project emissions from fuel consumption by project fuel trucks to carry CNG from the supplier of CNG to the CNG stations are excluded from project emissions since the emissions from fuel trucks to carry diesel fuel in reference scenario are considered to be the same as the ones for CNG in project scenario. |
| - Monitoring parameters | - CNG consumption by project buses - Diesel fuel consumption by project buses - Drive distance of project buses |

- Eligibility criteria

- This methodology is applicable to projects that satisfy all of the following criteria.

| Criterion 1 | CNG-diesel hybrid equipment is newly installed to the public transport buses which have already been in operation or are newly procured. |
| --- | --- |

- Emission Sources and GHG types

| Reference emissions |  |
| --- | --- |
| Emission sources | GHG types |
| Diesel fuel consumption by reference buses | CO2 |
| Project emissions |  |
| Emission sources | GHG types |
| Diesel fuel consumption by project buses | CO2 |
| CNG consumption by project buses | CO2 |

- Establishment and calculation of reference emissions

F.1. Establishment of reference emissions

Reference emissions are calculated with CNG consumption and diesel fuel consumption by project buses, net calorific value of CNG and diesel fuel, CO2 emission factor of diesel fuel, and fuel efficiency of project bus and reference bus. Fuel efficiency of project bus is determined ex-post based on monitored data, which reflect actual fuel efficiency. On the other hand, fuel efficiency of reference bus is determined ex-ante from the following three options in a conservative manner to ensure net emission reductions: [Option 1] Daily data sets of drive distance and diesel fuel consumption of bus i are collected prior to installation of CNG-diesel hybrid equipment. The highest value (the most efficient value) from the measured data sets for at least 60 days is selected and determined as fuel efficiency of reference bus i. [Option 2] A catalogue value of fuel efficiency of bus i which is converted from mono diesel fuel combustion to CNG-diesel hybrid combustion in the project is determined as fuel efficiency of reference bus i. A catalogue value usually shows better fuel efficiency than the one which is calculated for the bus being operated. Therefore, setting a default value of fuel efficiency of reference bus based on the catalogue values is conservative. [Option 3] The default value set in this methodology is applied as fuel efficiency of reference bus i. The default values are determined from the most recent catalogue values of public buses manufactured by Japanese manufacturers, which usually show better fuel efficiency than the ones which are calculated for the bus being operated, hence conservative.

F.2. Calculation of reference emissions

REp=iFCPJ,CNG,i,p×NCVCNG+FCPJ,diesel,i,p×NCVdiesel×ηPJ,i,pηRE,i×EFdiesel Where REp; Reference emissions during the period p [tCO2/p]. FCPJ,CNG,i,p; CNG consumption by project bus i during the period p [t/p]*. NCVCNG; Net calorific value of CNG [GJ/t]. FCPJ,diesel,i,p; Diesel fuel consumption by project bus i during the period p [kl/p]. NCVdiesel; Net calorific value of diesel fuel [GJ/kl]. ηPJ,i,p; Fuel efficiency of project bus i during the period p [km/l]. ηRE,i; Fuel efficiency of reference bus i [km/l]. EFdiesel; CO2 emission factor of diesel fuel [tCO2/GJ]. i; Identification number of project buses. *When CNG consumption by project bus is monitored in units of LSP (Litter Setara Premium), the value is converted in units of tonne [t] with the equation below. CNG consumption [t] = CNG consumption [LSP] × 0.7218 × 10-3 [Source] Ministry of Energy and Mineral Resources Fuel efficiency of project bus i during the period p (ηPJ,i,p) is calculated with the following equation. ηPJ,i,p=TDPJ,i,pHFCPJ,diesel,i,p×103 HFCPJ,diesel,i,p=iFCPJ,CNG,i,p×NCVCNG÷NCVdiesel+iFCPJ,diesel,i,p Where ηPJ,i,p; Fuel efficiency of project bus i during the period p [km/l]. TDPJ,i,p; Total drive distance of project bus i during the period p [km/p]. HFCPJ,diesel,i,p; Hypothetical total diesel fuel consumption by project bus i during the period p [kl/p]. FCPJ,CNG,i,p; CNG consumption by project bus i during the period p [t/p]. NCVCNG; Net calorific value of CNG [GJ/t]. NCVdiesel; Net calorific value of diesel fuel [GJ/kl]. FCPJ,diesel,i,p; Diesel fuel consumption by project bus i during the period p [kl/p]. i; Identification number of project buses.

- Calculation of project emissions

PEp=PECNG,p+PEdiesel,p PECNG,p=iFCPJ,CNG,i,p×NCVCNG×EFCNG PEdiesel,p=iFCPJ,diesel,i,p×NCVdiesel×EFdiesel Where PEp; Project emissions during the period p [tCO2/p]. PECNG,p; Project emissions from CNG consumption by project buses during the period p [tCO2/p]. PEdiesel,p; Project emissions from diesel fuel consumption by project buses during the period p [tCO2/p]. FCPJ,CNG,i,p; CNG consumption by project bus i during the period p [t/p]. NCVCNG; Net calorific value of CNG [GJ/t]. EFCNG; CO2 emission factor of CNG [tCO2/GJ]. FCPJ,diesel,i,p; Diesel fuel consumption by project bus i during the period p [kl/p]. NCVdiesel; Net calorific value of diesel fuel [GJ/kl]. EFdiesel; CO2 emission factor of diesel fuel [tCO2/GJ]. i; Identification number of project buses.

- Calculation of emissions reductions

ERp=REp-PEp Where ERp; Emission reductions during the period p [tCO2/p]. REp; Reference emissions during the period p [tCO2/p]. PEp; Project emissions during the period p [tCO2/p].

- Data and parameters fixed ex ante

The source of each data and parameter fixed ex ante is listed as below.

| Parameter | Description of data | Source |
| --- | --- | --- |
| NCVCNG | Net calorific value of CNG [GJ/t] | In the order of preference: a) value provided by fuel supplier; b) value measured by the project participants; c) regional or national default value; or d) IPCC default value provided in table 1.2 of Ch.1 Vol.2 of 2006 IPCC Guidelines on National GHG Inventories. Lower value is applied. |
| NCVdiesel | Net calorific value of diesel fuel [GJ/kl] | In the order of preference: a) value provided by fuel supplier; b) value measured by the project participants; c) regional or national default value; or d) IPCC default value provided in table 1.2 of Ch.1 Vol.2 of 2006 IPCC Guidelines on National GHG Inventories. Lower value is applied. |
| EFCNG | CO2 emission factor of CNG [tCO2/GJ] | In the order of preference: a) value provided by fuel supplier; b) value measured by the project participants; c) regional or national default value; or d) IPCC default value provided in table 3.2.1 of Ch.3 Vol.2 of 2006 IPCC Guidelines on National GHG Inventories. Higher value is applied. |
| EFdiesel | CO2 emission factor of diesel fuel [tCO2/GJ] | In the order of preference: a) value provided by fuel supplier; b) value measured by the project participants; c) regional or national default value; or d) IPCC default value provided in table 3.2.1 of Ch.3 Vol.2 of 2006 IPCC Guidelines on National GHG Inventories. Lower value is applied. |
| ηRE,i | Fuel efficiency of reference bus i [km/l] Fuel efficiency of reference bus is determined ex-ante in the following manner. [Option 1] Fuel efficiency of reference bus i is determined based on measured data of bus i prior to installation of CNG-diesel hybrid equipment. Daily data sets of drive distance and diesel fuel consumption of bus i are collected prior to installation of CNG-diesel hybrid equipment for at least 60 days. The highest value (the most efficient value) from the measured data sets is selected and determined as fuel efficiency of reference bus i. [Option 2] Catalogue value of fuel efficiency of bus i which is converted from mono diesel fuel combustion to CNG-diesel hybrid combustion in the project is determined as fuel efficiency of reference bus i. [Option 3] The default value in the following table in line with the total displacement is applied as fuel efficiency of reference bus i. (“x” in the table represents the total displacement of project bus i) Total displacement; x < 5.2L (5,200cc); 5.2L (5,200cc) ≤ x. ηRE,i; 6.5; 4.7. | [Option 1] Measured data. [Option 2] Catalogue values of fuel efficiency provided by bus manufacturer. [Option 3] The catalogues of public buses manufactured by Japanese manufacturers. The default value is revised if deemed necessary by the JC. |

History of the document

| Version | Date | Contents revised |
| --- | --- | --- |
| 01.0 | 23 December 2020 | Electronic decision by the Joint Committee Initial approval. |