Joint Crediting Mechanism Approved Methodology KE_AM003

“Installation of Run-of-river Small Hydropower Generation Plant”

- Title of the methodology

- Installation of Run-of-river Small Hydropower Generation Plant, Version 01.0

- Terms and definitions

| Terms | Definitions |
| --- | --- |
| Small hydropower plant | A small hydropower plant is a hydropower plant with an installed capacity of up to 10 MW. |
| Run-of-river hydropower plant | A run-of-river hydropower plant is a type of hydroelectric generation which does not have a dam. |

- Summary of the methodology

| - Items | - Summary |
| --- | --- |
| - GHG emission reduction measures | - Displacement of electricity using fossil fuel as a power source by installation and operation of a small hydropower plant. |
| - Calculation of reference emissions | The reference emissions are calculated on the basis of the net power output of the small hydropower plant which means the gross energy generation by the project small hydropower plant minus the auxiliary/station electricity consumption, multiplied by the conservative emission factor. |
| - Calculation of project emissions | - The project emissions are the emissions from the small hydropower plant, which are assumed to be zero. |
| - Monitoring parameters | - The quantity of electricity generated by the project small hydropower plant. |

- Eligibility criteria

- This methodology is applicable to projects that satisfy all of the following criteria.

| Criterion 1 | The project installs a run-of-river small hydropower plant. |
| --- | --- |

- Emission Sources and GHG types

| Reference emissions |  |
| --- | --- |
| Emission sources | GHG types |
| Generation of electricity using fossil fuel as power source | CO2 |
| Project emissions |  |
| Emission sources | GHG types |
| Generation of electricity from small hydropower plant | N/A |

- Establishment and calculation of reference emissions

F.1. Establishment of reference emissions

In order to identify the reference emission factor simplistically and secure net emission reductions, this methodology applies the lowest emission factor of diesel power generation. The most efficient diesel generator in the world has a generation efficiency close to 49%. A power generation efficiency of 49% translates into an emission factor of 0.533 tCO2/MWh. This value is lower than the lowest standardized grid emission factor in 2014, which is 0.5793 tCO2/MWh as the build margin calculated at that time addressed in the report published by the National Environment Management Authority of Kenya (NEMA 2014). This will ensure net emission reductions.

F.2. Calculation of reference emissions

REp : Reference emissions during period p [tCO2/p] EGp : Quantity of electricity generated by project small hydropower plant during period p [MWh/p] EFRE : Reference CO2 emission factor [tCO2/MWh]

- Calculation of project emissions

PEp = 0 PEp : Project emissions during period p [tCO2/p]

- Calculation of emissions reductions

ERp = REp － PEp = REp ERp : Emission reductions during period p [tCO2/p] REp : Reference emissions during period p [tCO2/p] PEp : Project emissions during period p [tCO2/p]

- Data and parameters fixed ex ante

The source of each data and parameter fixed ex ante is listed as below.

| Parameter | Description of data | Source |
| --- | --- | --- |
| EFRE | The reference CO2 emission factor The default value for EFRE is set to be 0.533 (tCO2/MWh). *The efficiency of the most efficient diesel engine is close to but below 49%. | Additional information. The default value is revised if deemed necessary by the JC. |

History of the document

| Version | Date | Contents revised |
| --- | --- | --- |
| 01.0 | 23 March 2017 | JC3, Annex 5 Initial approval. |

If the installed capacity of the project hydropower plant is equal to or less than 30 kW and not connected to national electricity grid on the day of validation, the approved methodology KE_AM001 “Electrification of communities using Micro hydropower generation” may be applied.

A dam is defined as a structure built across a river with a height greater than 15m or more from its foundation.