- Joint Crediting Mechanism Approved Methodology KH_AM004

“Reducing deforestation and forest degradation through forest conservation in Cambodia”

Title of the methodology

Reducing deforestation and forest degradation through forest conservation in Cambodia, version 1.1

Terms and definitions

| Terms | Definitions |
| --- | --- |
| Cambodia’s official forest reference level (National FRL) | Cambodia’s official forest reference level (National FRL) is defined as the most recent and available forest reference level submitted to the UNFCCC secretariat by Cambodia and completed technical assessment by the UNFCCC’s assessment team. |
| Cambodia’s official forest map | A land use and land cover map, endorsed as an official map, named Forest Cover, by the Government of Cambodia. The generation of additional landcover data are expected every 2 years, e.g., Forest Cover 2018 and Forest Cover 2020. |
| Forest classes | Classes of forests adopted in the establishment of Cambodia’s official forest reference level. |
| Transition probability | A probability that a forest class will be changed to another forest class, or be converted, to non-forest, based on the historical pace of transition. Transition probability is between 0 to 1. |

Summary of the methodology

| - Items | - Summary |
| --- | --- |
| Project activities (emission reduction measures) | The project activities include forest conservation activities such as forest management and community livelihood development, including agricultural improvement, eco-tourism development, marketing of non-timber forest products and formation/strengthening of community groups that lead to the reduction of emissions from deforestation and degradation. |
| Establishment of project reference level | Two options have been identified to establish the project reference level, using data from Cambodia’s official forest reference level (National FRL): - applying the National FRL transition probabilities from forest to non-forest classes only, to the project area, (e.g., transitions between different forest classes are not included); and - applying all the National FRL transition probabilities among classes that would result in emissions to the project area. Note that Option (1) accounts for emissions from only deforestation, while Option (2) accounts for emissions from forest degradation in form of transitions from one forest class to another, in addition to those from deforestation. Option 2 is similar to the method used in the Cambodia’s National FRL submission in 2017, except that the National FRL includes removals as well; this methodology targets only emissions. |
| Calculation of project net emissions | Project net emissions are estimated based on the result of monitoring of forest to non-forest classes (Option 1) and monitoring of forest to non-forest or another forest class (Option 2) within the project area, multiplied by the emission factors (including both above and below ground biomass) plus any displaced emissions within the displacement belt and GHG emissions due to the project activities inside the project area and the activity area in line with Joint Crediting Mechanism Guidelines for Developing Proposed Methodology for Reducing Emissions from Deforestation and Forest Degradation, and the Role of Conservation, Sustainable Management of Forests and Enhancement of Forest Carbon Stocks in Developing Countries (REDD-plus), (methodology guidelines). Displaced emissions are calculated as increases of emissions in the displacement belt compared to the reference emissions. Use of fuel and fertilizers, for the implementation of the project activities, are included as GHG emissions sources, and will be monitored and accounted as project emissions. |
| Monitoring parameters and methods | - Area converted from forest to non-forest (Option 1) or forest to another forest class (Option 2) in the project area - Area converted from forest to non-forest (Option 1) or forest to another forest class (Option 2) in the displacement belt - Quantity of fuel consumed in vehicle and/or equipment - Number of vehicle and/or equipment - Total travel distance of vehicle and/or total use hours of equipment - Average specific energy consumption of vehicle and/or equipment - Mass of synthetic fertilizer applied - Mass of organic fertilizer applied and made from materials sourced from outside of the project area and the activity area - Harvested annual dry matter yield of N-fixing crop per unit area, introduced by the project - Total annual area harvested of N-fixing crop introduced by the project - Ratio of above-ground residues to harvested yield for N-fixing crop - Ratio of below-ground residues to harvested yield for N-fixing crop - Mass of calcic limestone, dolomite and urea fertilizer applied - Nitrogen content of synthetic and organic fertilizer applied - Nitrogen content of above-ground and below-ground residues for N-fixing crop - Fraction of total area under N-fixing crop that is renewed annually |
| Calculation of project emission reductions to be credited | A default discount factor of 20%, as defined in the methodology guidelines, is applied to project emissions reductions to account for the risk of reversal. |

Eligibility criteria

- This methodology is applicable to projects that satisfy all of the following criteria.

| Criterion 1 | The project is to reduce deforestation and forest degradation through project activities including forest management and community livelihood development. |
| --- | --- |
| Criterion 2 | Cambodia’s official forest reference (emission) level has been submitted to UNFCCC, completed technical assessment by UNFCCC, and is publicly available. |
| Criterion 3 | Cambodia’s official forest map for the project start year or less than or equal to two years old is available for the project participant. |
| Criterion 4 | Project activities do not include activities which lead to GHG emissions within the project area and the project activity area, except for the use of fuel or fertilizer including N-fixing crops. |

Geographical Boundaries

Essential

| Geographical boundary | Requirements |
| --- | --- |
| Project area | No requirements in addition to those described in paragraphs 17 to 22 of the methodology guidelines ver01.0 (JCM_KH_GL_PM_REDD+_ver01.0) |
| Reference area | The land area of Cambodia is used to keep consistency with National FRL. |

Optional boundaries

| Geographical boundary | Required (Y/N/TBD) | Additional requirements |
| --- | --- | --- |
| Activity area | Y | No requirements in addition to those described in paragraph 17 and 21 of the methodology guidelines ver01.0. |
| Displacement belt | Y | Displacement belt is the forest which is located outside of the project area, where deforestation and forest degradation could occur due to the displacement of project activities, such as small-scale logging, agricultural encroachment, and collection of non-timber forest products. The displacement belt is delineated on the basis of information on the impact of project activities which are obtained from local experts and other sources. If other REDD+ projects exist within the belt, project areas of these projects are excluded from the displacement belt. If development plans, including Economic Land Concessions (ELCs), exist within the belt, those areas are also excluded because forest clearance is likely planned inside these areas regardless the project activities. |

TBD: to be decided by the project participant

Carbon pools and GHG sources

The net emission sources to be considered include all the following Carbon pools and GHG sources.

| Project reference level |  |  |  |
| --- | --- | --- | --- |
| Carbon pools and GHG sources |  | Included (Y/N) | Explanation |
| Carbon pools | Above ground biomass | Y | This pool is expected to contribute significantly to emissions and emission reductions and is therefore included. |
|  | Below ground biomass | Y | This pool is expected to contribute significantly to emissions and emission reductions and is therefore included. |
|  | Dead wood | N | It is expected that this pool would have decreased in the absence of the project and, therefore, it is conservatively excluded. |
|  | Litter | N | It is expected that this pool would have decreased in the absence of the project and, therefore, it is conservatively excluded. |
|  | Soil organic carbon | N | It is expected that this pool would have decreased in the absence of the project and, therefore, it is conservatively excluded. |
| GHG sources |  |  |  |
| Project net emissions |  |  |  |
| Carbon pools and GHG sources |  | Included (Y/N) | Explanation |
| Carbon pools | Above ground biomass | Y | This pool is expected to contribute significantly to emissions and emission reductions and is therefore included. |
|  | Below ground biomass | Y | This pool is expected to contribute significantly to emissions and emission reductions and is therefore included. |
|  | Dead wood | N | It is expected that this pool would have decreased in the absence of the project and, therefore, it is conservatively excluded. |
|  | Litter | N | It is expected that this pool would have decreased in the absence of the project and, therefore, it is conservatively excluded. |
|  | Soil organic carbon | N | It is expected that this pool would have decreased in the absence of the project, and therefore, it is conservatively excluded. |
| GHG sources | CO2 emissions from combustion of fossil fuels |  | This GHG source is estimated if the project activities which include combustion of fossil fuels are implemented. |
|  | N2O and CO2 emissions from fertilizer application |  | This GHG source is estimated if the project activities include fertilizer application. |

Establishment of project reference level

G.1. Establishment of project reference level

| Approach for estimating project reference level | In order to maximize consistency, the project reference level is established by applying emission factors and transition probabilities from the national forest reference level (National FRL). |
| --- | --- |

Reference area and reference period are the same as the National FRL, and therefore the reference area is the total of Cambodia’s land area. The reference period is the same as the reference period submitted in the National FRL to the UNFCCC. In the National FRL submitted in 2017; the reference period was 2006 to 2014. Calculation of transition probability is done by applying one of two options provided in this methodology: (1) Option 1: using the National FRL transition probabilities from forest to non-forest classes only (i.e., transitions between different forest classes are not included); or (2) Option 2: using all the National FRL transition probabilities among classes that would result in GHG emissions. The option selected should also be used for calculation of project net emissions in section H and for determining data and parameters fixed ex ante in section J. The option is selected in coordination with Government of Cambodia and project participants. Yearly reference emission at the project reference level is calculated by applying the transition probabilities to areas of forests at the previous year and multiplying by emission factors. To ensure the conservativeness of the calculation of the project emission reductions, dead wood, litter and soil organic carbon pools, of which carbon stocks would have decreased in the absence of the project, were excluded.

G.2. Calculation of project reference level

The project reference level in year y during the monitoring period is calculated as follows: RLy = ∆CSref y * 44/12 Equation 1 Where: RLy Project reference level in year y; tCO2 ∆CSref y Projected carbon stock change in the project area in year y; tC 44/12 Conversion factor of molecular weight of carbon to CO2; dimensionless ∆CSref y is calculated by applying either Option 1 or Option 2 selected in coordination with government of Cambodia and project participants. Option 1: Use the National FRL transition probabilities from forest to non-forest classes only Carbon stock change in the project area in year y is calculated as follows: For the project start year: ∆CSref y = i(Ai 0 * Pi * dy /d0 y * EFi) Equation 2 For subsequent years to the project start year: ∆CSref y = i(Ai y-1 * Pi * dy /d0 y * EFi) Equation 3 Where: ∆CSref y Projected carbon stock change in the project area in year y; tC Ai 0 Area of forest class i in the project area at the inception of the project; ha Ai y-1 Area of forest class i in the project area in year y-1; ha i Forest class in the project area; dimensionless Pi Annual transition probability from forest class i to non-forest within the reference area; dimensionless, 0-1 dy Number of operating days in year y, days do y Number of days in year y, days EFi Emission factor applicable for forest class i; tC ha-1 Area of forest class i is assumed to decrease every year due to deforestation, and therefore calculated as follows: For the project start year: Ai y = Ai 0 * (1 - Pi * dy /d0 y ) Equation 4 For subsequent years to the project start year: Ai y = Ai y-1 * (1 - Pi) Equation 5 Where: Ai y Area of forest class i in the project area in year y; ha Ai 0 Area of forest class i in the project area at the inception of the project; ha Ai y-1 Area of forest class i in the project area in year y-1; ha Pi Annual transition probability from forest class i to non-forest within the reference area; dimensionless, 0-1 dy Number of operating days in year y, days do y Number of days in year y, days Please refer to section J for determination of EFi, Ai 0 (area at the inception of the project) and Pi. Option 2: Use all the National FRL transition probabilities among classes that would result in GHG emissions. To estimate the total emissions, the projected carbon stock change in year y in the project area is calculated as follows: ∆CSref y=ijcsij ycsij y , if csij y>00, otherwise Equation 6 Where: ∆CSref y Projected carbon stock change in the project area in year y; tC csij y Projected carbon stock change in the project area from changes of land use category i to j in year y; tC i, j Land use category in the project area; dimensionless The notation csij y is an element of i-by-j cross-tabulation matrix MCSy which is a product of element-wise multiplication (Hadamard product) of i-by-j matrices of area changes and emission factors. MCSy= MCAy ⋅ MEF Equation 7 Where: MCSy i-by-j matrix in which each element is projected carbon stock change in the project area from changes of land use category i to j in the project area in year y, csij y; tC MCAy i-by-j matrix in which each element is projected area of land converted from land use category i to j in the project area in year y, caij y; ha MEF i-by-j matrix in which each element is emission factor for area of land converted from land use category i to j, EFij; tC ha-1 Equation 5 is expressed as below: cs11ycs12y …cs1jycs21ycs22y…cs2jy…………csi1ycsi2y…csijy=ca11yca12y …ca1jyca21yca22y…ca2jy…………cai1ycai2y…caijy∙EF11EF12 …EF1jEF21EF22…EF2j…………EFi1EFi2…EFij For example, the cs11 y is calculated as bellow: cs11 y = ca11 y * EF11 Projected area of land conversion in year y is calculated as the product between areas of land categories in the project area in year y-1 and annual transition probabilities. MCAy= MAy-1MP Equation 8 Where: MCAy i-by-j matrix in which each element is the projected area of land converted from land use category i to j in year y, caij y; ha MAy-1 i-by-j diagonal matrix whose diagonal elements are areas of land use categories i in the project area in year y-1, Ai y-1; ha MP i-by-j matrix in which each element is the annual transition probability from land use category i to j, pij; dimensionless, 0-1 Equation 6 is expressed as below: ca11yca12y …ca1jyca21yca22y…ca2jy…………cai1ycai2y…caijy= A1y-10 …00A2y-1…0…………00…Aiy-1p11p12 …p1jp21p22…p2j…………pi1pi2…pij For example, selected elements are calculated as bellow: ca11y = A1y-1 * p11 + 0 * p21 +… + 0 * pi1 ca21y = 0* p11 + A2y-1 * p21 +… + 0 * pi1 ca22y = 0 * p12 + A2y-1 * p22 +… + 0 * pi2 caijy = 0 * p1j + 0 * p2j +… + Aiy-1 * pij Area of each land use category is decreased due to transition to other land categories and increased due to transition from other land use categories. In year y+1, the area of a land use category is the sum of the area staying in the same land use category (e.g., ca11y) plus the area that transitioned to that land use category from other land use categories (e.g., ca21 y, ca31 y, and cai1 y). Area of land use category j in year y+1, Aj y+1, is calculated as follows: Aj y+1= icaij y Equation 9 Where: Aj y+1 Area of land use category j in project area in year y+1; ha caij y Area of land converted from land use category i to j in the project area in year y; ha Please refer to section J for determination of EFij, Ai 0 and pij, i.e., MEF, MA0 and MP.

- Calculation of project net emissions

Project net emissions in year y during the monitoring period are estimated based on results of monitoring, and calculated as follows: PEy = ∆CSpj y * 44/12 + Efuel y + Efertilizer y + DEy Equation 10 Where: PEy Project net emissions in year y; tCO2-eq ∆CSpj y Carbon stock change in the project area in year y; tC Efuel y CO2 emissions from fossil fuel combustion in year y due to the project activities; tCO2 Efertilizer y GHG emissions from fertilizer application within the project area and the activity area as a part of the project activities in year y; tCO2-eq DEy Displaced emissions to the displacement belt in year y; tCO2 Carbon stock change in the project area in year y, ∆CSpj y The same option applied in the calculation of the project reference level must be used for the project emissions. Option 1: Use the National FRL transition probabilities from forest to non-forest classes only Carbon stock change in the project area in year y is calculated as follows: ∆CSpj y = i∆CSpj i y = iCApj i y *EFi Equation 11 Where: ∆CSpj y Carbon stock change in the project area in year y; tC ∆CSpj i y Carbon stock change in area converted from forest class i to non-forest in the project area in year y; tC CApj i y Area converted from forest class i to non-forest in the project area in year y; ha EFi Emission factor applicable for forest class i; tC ha-1 CApj i y will be determined using Cambodia’s official forest maps provided by the government; calculated as the yearly average, if the monitoring interval is more than one year. Accuracy assessment: to better account for the uncertainty that may be present in Cambodia’s official forest map within the project area, an accuracy assessment will be performed of the project area and displacement belt. The accuracy assessment will be performed on the most recent official forest map and using a combination of the points used by the government of Cambodia and with additional points to ensure representative coverage of all classes within the project area. The results of the accuracy assessment of the project area and the displacement belt will determine the error-adjusted area estimates for each forest class. The accuracy assessment will be performed at the start of the project and each subsequent monitoring period to ensure high quality estimates of emission and reductions. This process is based on the methods included in Olofsson et al. 2014 and is endorsed by the FAO. However, the error adjustment of the areas will not be performed, if the government of Cambodia decides to require the project to use Cambodia’s official forest map as it is. See section J for EFi. Option 2: Use all the National FRL transition probabilities among classes that would result in GHG emissions To estimate the total emissions, the carbon stock change in the project area in year y is calculated as follows: ∆CSpj y=ijcspj ij ycspj ij y , if cspj ij y>00, otherwise Equation 12 Where: ∆CSpj y Carbon stock change in the project area in year y; tC cspj ij y Carbon stock change in the project area from changes of land use category i to j in year y; tC The notation cspj ij y is an element of i-by-j cross-tabulation matrix MCSpj y which is a product of element-wise multiplication (Hadamard product) of i-by-j matrices of area changes and emission factors. MCSpj y= MCApj y ⋅ MEF Equation 13 Where: MCSpj y i-by-j matrix in which each element is carbon stock change in the project area from changes in land use category i to j in year y, cspj ij y; tC MCApj y i-by-j matrix in which each element is the area of land converted from land use category i to j in the project area in year y, capj ij y; ha capj ij y Area of land converted from land use category i to j in the project area in year y; ha MEF i-by-j matrix in which each element is emission factor for area of land converted from land use category i to j, EFij; tC ha-1 capj ij y will be determined using Cambodia’s official forest maps provided by the government; calculated as the yearly average, if the monitoring interval is more than one year. To quantify and account for the uncertainty of Cambodia’s official forest map in the project area, an accuracy assessment will be performed as described above. See section J for EFij. Emissions due to the project activities Based on the requirements of the methodology guidelines ver01.0, emissions resulting from the implementation of the project activities shall be accounted, including fossil fuel combustion and the application of fertilizer (synthetic fertilizer, organic fertilizer, N-fixing crop, liming material and urea fertilizer). If the GHG emissions, due to use of fuel or the application of fertilizers, are estimated to less than five percent of the total net emissions at the time of validation, quantities of such GHG sources do not need to be monitored, and planned quantities can be used for calculation of GHG emissions. Emissions from fossil fuel combustion CO2 emissions from fossil fuel combustion resulting from the implementation of project activities are calculated by applying the following direct method or indirect method, and these can be used interchangeably or simultaneously for different types of vehicle and equipment. E.g., project participant can calculate Efuel y by applying the direct method for motorbikes and the in-direct method for trucks. Direct method Efuel y=fEfuel f y =f(FCf y *NCVf * EFfuel f) Equation 14 Where: Efuel y CO2 emissions from fossil fuel combustion in year y due to the project activities; tCO2 Efuel f y CO2 emissions from combustion of fossil fuel type f in year y; tCO2 FCf y Quantity of fuel type f consumed in year y; kg NCVf Net calorific value of fuel f; GJ kg-1 EFfuel f CO2 emission factor of the fuel type f combusted; tCO2 GJ-1 f fuel types combusted; dimensionless See section J for NCVf and EFfuel f. Indirect method Efuel y=fjEfuel j f y =fj(NVEj f y *TDUj f y * SECj f * NCVf* EFfuel f) Equation 15 Where: Efuel y CO2 emissions from fossil fuel combustion in year y due to the project activities; tCO2 Efuel j f y CO2 emissions from fossil fuel combustion in vehicle or equipment type j using fuel type f in year y; tCO2 NVEj f y Number of vehicle or equipment type j using fuel type f in year y; unit TDUj f y Total travel distance for vehicle type j or use hours for equipment type j using fuel type f in year y; km or hour unit-1 SECj f Average specific energy consumption of vehicle or equipment type j for fuel type f; kg km-1 or hour-1 NCVf Net calorific value of fuel f; GJ kg-1 EFfuel f CO2 emission factor of the fuel type f combusted; tCO2 GJ-1 f fuel types combusted; dimensionless j type of vehicle or equipment; dimensionless For NCVf, EFfuel f, and SECj f, see section J. Emissions from fertilizer application GHG emissions from fertilizer application are calculated as follow: Efertilizer y = Edirect-N y + Eindirect-N y + Eliming y + Eurea y Equation 16 Where: Efertilizer y GHG emissions from fertilizer application within the project area and the activity area for implementation of the project activities in year y; tCO2-eq Edirect-N y Direct N2O emissions as a result of nitrogen application within the project area and the activity area for implementation of the project activities in year y; tCO2-eq Eindirect-N y Indirect N2O emissions as a result of nitrogen application within the project area and the activity area for implementation of the project activities in year y; tCO2-eq Eliming y CO2 emissions as a result of adding liming materials within the project area and the activity area for implementation of the project activities in year y; tCO2 Eurea y CO2 emissions as a result of urea fertilization application within the project area and the activity area for implementation of the project activities in year y; tCO2 Direct N2O emissions as a result of nitrogen application for the implementation of the project activities are calculated as follow: Edirect-N y =c[FSN c y+ FON c y+ FCR c y *EFdirect-N c] * 4428 *GWPN2O Equation 17 Where: Edirect-N y Direct N2O emissions as a result of nitrogen application within the project area and the activity area for implementation of the project activities in year y; tCO2-eq FSN c y Mass of nitrogen in synthetic fertilizer applied for implementation of the project activities in cropland type c in the project area and the activity area in year y; tN FON c y Mass of nitrogen in organic fertilizer made from materials sourced from outside of the project area and the activity area and applied for implementation of the project activities in cropland type c in the project area and the activity area in year y; tN FCR c y Mass of nitrogen in crop residues (above-ground and below-ground) in N-fixing crops, introduced for implementation of the project activities in cropland type c in the project area and the activity area and returned to soils, in year y; tN EFdirect-N c Emission factor for N2O emissions from nitrogen inputs in cropland type c; t N2O-N tN-input-1 44/28 Ratio of molecular weight of N2O and N; dimensionless GWPN2O Global Warming Potential for N2O; tCO2 (t N2O)-1 c Types of croplands: upland cropland and flooded cropland such as rice paddy; dimensionless See section J for EFdirect-N c and GWPN2O. FSN c y= MSN c y * NCSN c Equation 18 FON c y= MON c y * NCON c Equation 19 Where: FSN c y Mass of nitrogen in synthetic fertilizer applied for implementation of the project activities in cropland type c in the project area and the activity area in year y, tN FON c y Mass of nitrogen in organic fertilizer made from materials sourced from outside of the project area and the activity area and applied for implementation of the project activities in cropland type c in the project area and the activity area in year y, tN MSN c y Mass of synthetic fertilizer applied for implementation of the project activities in cropland type c in the project area and the activity area in year y, t MON c y Mass of organic fertilizer made from materials sourced from outside of the project area and the activity area and applied for implementation of the project activities in cropland type c in the project area and the activity area in year y, t NCSN c Nitrogen content of synthetic fertilizer applied in cropland type c; tN (t fertilizer) -1 NCON c Nitrogen content of organic fertilizer applied in cropland type c; tN (t fertilizer) -1 Data from producers of synthetic fertilizer and published data are used for NCSN c NCON c, respectively. If multiple types of synthetic or organic fertilizers are used, choose the highest data as a conservative estimation. Note that organic fertilizer which is made from organic materials sourced from inside the project area and the activity area are NOT accounted because the emissions from those organic materials occur in the areas regardless the implementation of the project activities. FCR c y= TCropc T y * Areac T y * FracRenew T * RAG T *NAG T + RBG T * NBG T Equation 20 Where: FCR c y Mass of nitrogen in crop residues (above-ground and below-ground) in N-fixing crops, introduced for implementation of the project activities in cropland type c in the project area and the activity area and returned to soils, in year y; tN Cropc T y Harvested annual dry matter yield for N-fixing crop T per unit area, introduced for implementation of the project activities in cropland type c in the project area and the activity area in year y; t d.m. ha -1 Area c T y Total annual area harvested of N-fixing crop T, introduced for implementation of the project activities in cropland type c in the project area and the activity area in year y; ha FracRenew T Fraction of total area under N-fixing crop T that is renewed annually; dimensionless, 0-1 RAG T Ratio of above-ground residues to harvested yield for N-fixing crop T; t d.m. (t d.m.)-1 NAG T N content of above-ground residues for N-fixing crop T; t N (t d.m.)-1 RBG T Ratio of below-ground residues to harvested yield for N-fixing crop T; t d.m. (t d.m.)-1 NBG T N content of below-ground residues for N-fixing crop T; t N (t d.m.)-1 T Types of N-fixing crops; dimensionless Where cropland is renewed on average every X years, Fracrenew = 1/X. For annual crops Fracrenew = 1. Indirect N2O emissions as a result of nitrogen application for implementation of the project activities are calculated as follow: Eindirect-N,y = FSN y * FracSN + FON y * FracON * EFindirect-N + FSN y+ FON y+ FCR y * Fracleach *EFleach-N* 4428 * GWPN2O Equation 21 Where: Eindirect-N y Indirect N2O emissions as a result of nitrogen application within the project area and the activity area for implementation of the project activities in year y; tCO2-eq FSN y Mass of nitrogen in synthetic fertilizer applied for implementation of the project activities in the project area and the activity area in year y; tN FON y Mass of nitrogen in organic fertilizer made from materials sourced from outside the project area and the activity area and applied for implementation of the project activities in the project area and the activity area in year y; tN FCR y Mass of nitrogen in crop residues (above-ground and below-ground) in N-fixing crops, introduced for implementation of the project activities in the project area and the activity area and returned to soils, in year y; tN FracSN Fraction that volatilized as NH3 and NOX for synthetic fertilizers; dimensionless, 0-1 FracON Fraction that volatilized as NH3 and NOX for organic fertilizers; dimensionless, 0-1 EFindirect-N Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces; t N2O-N (t NH3-N and NOX-N volatilized)-1 Fracleach Fraction of N that is lost through leaching and runoff; dimensionless, 0-1 EFleach-N Emission factor for N2O emissions from N leaching and runoff; t N2O-N (t N leaching and runoff)-1 44/28 Ratio of molecular weight of N2O and N; dimensionless GWPN2O Global Warming Potential for N2O; tCO2 (t N2O) -1 See Section J for FracSN, FracON, Fracleach, EFindirect-N, EFleach-N and GWPN2O. FSN y= cFSN c y Equation 22 FON y= cFON c y Equation 23 FCR y= cFCR c y Equation 24 Where: FSN y Mass of nitrogen in synthetic fertilizer applied for implementation of the project activities in the project area and the activity area in year y; tN FON y Mass of nitrogen in organic fertilizer made from materials sourced from outside the project area and the activity area and applied for implementation of the project activities in the project area and the activity area in year y; tN FCR y Mass of nitrogen in crop residues (above-ground and below-ground) in N-fixing crops, introduced for implementation of the project activities in the project area and the activity area and returned to soils, in year y; tN FSN c y Mass of nitrogen in synthetic fertilizer applied for implementation of the project activities in cropland type c in the project area and the activity area in year y; tN FON c y Mass of nitrogen in organic fertilizer made from materials sourced from outside the project area and the activity area and applied for implementation of the project activities in cropland type c in the project area and the activity area in year y; tN FCR c y Mass of nitrogen in crop residues (above-ground and below-ground) in N-fixing crops, introduced for implementation of the project activities in cropland type c in the project area and the activity area and returned to soils, in year y; tN Use FSN c y, FON c y and FCR c y calculated in Equations 16, 17 and 18. CO2 emissions as a result of adding liming materials for implementation of the project activities are calculated as follow: Eliming y= Mlimestone y * EFlimestone+ Mdolomite y * EFdolomite * 4412 Equation 25 Where: Eliming y CO2 emissions as a result of adding liming materials within the project area and the activity area during implementation of the project activities in year y; tCO2 Mlimestone y Mass of calcic limestone (CaCO3) applied for implementation of the project activities in the project area and the activity area in year y; t EFlimestone Emission factor for limestone; t C (t limestone)-1 Mdolomite y Mass of dolomite (CaMg(CO3)2) applied for implementation of the project activities in the project area and the activity area in year y; t EFdolomite Emission factor for dolomite; t C (t dolomite) -1 44/12 Ratio of molecular weight of CO2 and C; dimensionless See Section J for EFlimestone and EFdolomite. CO2 emissions as a result of urea fertilization application for implementation of the project activities are calculated as follow: Eurea y= Murea y* EFurea * 4412 Equation 26 Where: Eurea y CO2 emissions as a result of application of urea within the project area and the activity area for implementation of the project activities in year y; tCO2 Murea y Mass of urea fertilizer applied for implementation of the project activities in the project area and the activity area in year y; t EFurea Emission factor for urea; t C/t urea 44/12 Ratio of molecular weight of CO2 and C; dimensionless See Section J for EFurea. Displaced emissions In this methodology, displacement is assessed through monitoring of the displacement belt. The displacement belt captures the displacement of baseline activities due to the project activities, or interventions, in the project area. Although a detailed analysis of the local drivers of deforestation and degradation is not anticipated, the local context will be captured through inputs from local and national experts including authorities. Displaced emissions are calculated as increases of emissions compared to reference emissions from the displacement belt which is separately calculated from reference emissions for the project reference level. Displaced emissions are calculated as follows: DEy=DPy-DRy, if DPy-DRy>00, otherwise Equation 27 Where: DEy Displaced emissions to the displacement belt in year y; tCO2 DRy Reference emissions from the displacement belt in year y; tCO2 DPy Project emissions from the displacement belt in year y; tCO2 DRy and DPy are calculated as follows: DRy = ∆CSd y * 44/12 Equation 28 Where: DRy Reference emissions from the displacement belt in year y; tCO2 ∆CSd y Projected carbon stock change in the displacement belt in year y; tC 44/12 Conversion factor of molecular weight of carbon to CO2; dimensionless DPy = ∆CSd pj y * 44/12 Equation 29 Where: DPy Project emissions from the displacement belt in year y; tCO2 ∆CSd pj y Actual carbon stock change in the displacement belt in year y; tC 44/12 Conversion factor of molecular weight of carbon to CO2; dimensionless Projected and actual carbon stock changes in the displacement belt area, i.e., ∆CSd y and ∆CSd pj y, are calculated by applying the option used in Section G.2 Calculation of project reference level. Option 1: Use the National FRL transition probabilities from forest to non-forest classes only Projected carbon stock change in the displacement belt in year y is calculated as follows: For the project start year: ∆CSd y = iAd i 0 * Pd i * dy /d0 y * EFi Equation 30 For subsequent years to the project start year ∆CSd y = iAd i y-1 * Pd i * dy /d0 y * EFi Equation 31 Where: ∆CSd y Projected carbon stock change in the displacement belt in year y; tC Ad i 0 Area of forest class i in the displacement belt at the inception of the project; ha Ad i y-1 Area of forest class i in the displacement belt in year y-1; ha Pd i Annual transition probability from forest class i to non-forest within the displacement belt; dimensionless, 0-1 dy Number of operating days in year y, days do y Number of days in year y, days EFi Emission factor applicable for forest class i; tC ha-1 Area of forest class i is assumed to decrease every year due to deforestation, and therefore calculated as follows: For the project start year: Ad i y = Ad i 0 * (1 - Pd i * dy /d0 y ) Equation 32 For subsequent years to the project start year: Ad i y = Ad i y-1 * (1 - Pd i) Equation 33 Where: Ad i y Area of forest class i in the displacement belt in year y; ha Ad i 0 Area of forest class i in the displacement belt at the inception of the project; ha Ad i y-1 Area of forest class i in the displacement belt in year y-1; ha Pd i Annual transition probability from forest class i to non-forest within the displacement belt; dimensionless, 0-1 dy Number of operating days in year y, days do y Number of days in year y, days Please refer section J for Ad i 0 and Pd i. Actual carbon stock change in the displacement belt in year y is calculated as follows: ∆CSd pj y = ∑CAd pj i y * EFi Equation 34 Where: ∆CSd pj y Actual carbon stock change in the displacement belt in year y; tC CAd pj i y Area converted from forest class i to non-forest in the displacement belt in year y; ha EFi Emission factor applicable for land use category i; tC ha-1 CAd pj i y will be determined using Cambodia’s official forest maps provided by the government; calculated as yearly average, if the monitoring interval is more than one year. Where evidence can be collected that deforestation in the displacement belt is not attributable to the project, the detected deforestation is not considered as displacement and therefore is excluded from CAd pj i y. To quantify and account for the uncertainty of Cambodia’s official forest map in the project area, an accuracy assessment will be performed as described in Section H calculation of project net emission, sub-section (1) carbon stock change in the project area. See section J for EFi. Option 2: Use all transition probabilities resulting in emissions in the National FRL Projected carbon stock change in the displacement belt in year y is calculated as follows: To estimate the total emissions, the projected carbon stock change in year y in the displacement belt is calculated as follows: ∆CSd y=ijcsd ij ycsd ij y , if csd ij y>00, otherwise Equation 35 Where: ∆CSd y Projected carbon stock change in the displacement belt in year y; tC csd ij y Projected carbon stock change in the displacement belt from changes of land use category i to j in year y; tC The notation csd ij is an element of i-by-j cross-tabulation matrix MCSd y which is a product of element-wise multiplication (Hadamard product) of i-by-j matrices of area changes and emission factors. MCSd y= MCAd y ⋅MEF Equation 36 Where: MCSd y i-by-j matrix in which each element is projected carbon stock change in the displacement belt from changes of land use category i to j in year y, csd ij y; tC MCAd y i-by-j matrix in which each element is projected area of land converted from land use category i to j in the displacement belt in year y, cad ij y; ha MEF i-by-j matrix in which each element is the emission factor for area of land converted from land use category i to j, EFij; tC ha-1 Projected area of land conversion in year y is calculated as the product between areas of land categories in the displacement belt in year y and annual transition probability. MCAd y= MAd y-1MPd Equation 37 Where: MCAd y i-by-j matrix in which each element is the projected area of land converted from land use category i to j in the displacement belt in year y, cad ij y; ha MAd y-1 i-by-j diagonal matrix whose diagonal elements are areas of land use categories in the displacement belt in year y-1, Ad i y-1; ha MPd i-by-j matrix in which each element is the annual transition probability from land use category i to j in the displacement belt, pd ij; dimensionless, 0-1 Similar to Equation 7, area of land use category j in year y+1, Ad j y+1, is calculated as follows: Ad j y+1 =icad ij y Equation 38 Where: Ad j y+1 Area of land use category j in the displacement belt in year y+1; ha cad ij y Area of land converted from land use category i to j in the displacement belt in year y; ha Please refer section J for determination of EFij, Ad i 0 and pd ij, i.e., MEF, MAd 0 and MPd. Actual carbon stock change in the displacement belt in year y is calculated as follows: To estimate the total emissions, the carbon stock change in the displacement belt in year y is calculated as follows: ∆CSd pj y=ijcsd pj ij ycsd pj ij y , if csd pj ij y>00, otherwise Equation 39 Where: ∆CSd pj y Actual carbon stock change in the displacement belt in year y; tC csd pj ij y Carbon stock change in the displacement belt from changes of land use category i to j in year y; tC The notation csd pj ij is an element of the i-by-j cross-tabulation matrix MCSd pj y which is a product of element-wise multiplication (Hadamard product) of i-by-j matrices of area changes and emission factors. MCSd pj y= MCAd pj y ⋅MEF Equation 40 Where: MCSd pj y i-by-j matrix in which each element is carbon stock change in the displacement belt from changes of land use category i to j in year y, csd pj ij y; tC MCAd pj y i-by-j matrix in which each element is area of land converted from land use category i to j in the displacement belt in year y, cad pj ij y; ha cad pj ij y Area of land converted from land use category i to j in the displacement belt in year y; ha MEF i-by-j matrix in which each element is emission factor for area of land converted from land use category i to j, EFij; tC ha-1 cad pj ij y will be determined by using Cambodia’s official forest maps provided by the government, will be calculated as yearly average, if the monitoring interval is more than one year. Where evidence can be collected that deforestation in the displacement belt is not attributable to the project, the detected deforestation is not considered as displacement and therefore is excluded from MCAd pj y. To quantify and account for the uncertainty of Cambodia’s official forest map in the project area, an accuracy assessment will be performed as described in Section H calculation of project net emission, sub-section (1) carbon stock change in the project area. See section J for EFij.

Calculation of project emission reductions to be credited

Project emission reductions in year y are calculated as the difference between the project reference level and the project net emissions. ERy = RLy – PEy Equation 41 Where: ERy Project emissions reductions in year y; tCO2-eq RLy Project reference level in year y; tCO2 PEy Project net emissions in year y; tCO2-eq ERcredt y = ERy * (1-DF) Equation 42 Where: ERcredit y, Project emissions reductions available to be credited in year y; tCO2-eq ERy Project emissions reductions in year y; tCO2-eq DF Discount factor, default as 0.2 A discount factor is applied as a measure for risk of reversals. Project emissions reductions available to be credited for a monitoring period are calculated as follows: ERcredit p= iERcredit y Equation 43 Where: ERcredit p Project emissions reductions available to be credited for a monitoring period p; tCO2-eq ERcredit y, Project emissions reductions available to be credited in year y; tCO2-eq

Data and parameters fixed ex ante

The lists of the source of each data and parameter fixed ex ante and the source of each of default values selected in this methodology are available at the end of this section. The same option applied in the calculation of project reference level must be used. - Emission factors for land conversions Emission factors are calculated by applying carbon stock data used in National FRL, or other official report by Cambodian government. Table 1 shows carbon stock data used in the National FRL submitted in 2017. As new official data becomes available, emission factors can be updated in coordination with government of Cambodia. Table 1 Above ground and below ground biomass data used in the National FRL submitted in 2017 Forest/Non-Forest; Land use category; Abbreviation; Above-ground, tCha-1; Below-ground, tCha-1; Total biomass, tCha-1. Forest; Evergreen forest; E; 76.61; 14.69; 91.30. Semi-evergreen forest; SE; 114.21; 20.9; 135.11. Pine forest; P; 47; 9.54; 56.54. Deciduous forest; D; 39.95; 8.26; 48.21. Bamboo; B; 0; 0; 0. Mangrove; M; 70.5; 13.65; 84.15. Rear Mangrove; MR; 77.55; 14.85; 92.40. Flooded forest; FF; 32.9; 6.96; 39.86. Forest regrowth; FR; 35.25; 7.4; 42.65. Tree plantation; TP; 47; 9.54; 56.54. Pine plantation; PP; 47; 9.54; 56.54. Non-forest; Non-forest; NF; 0; 0; 0. Although Cambodia defined tree plantation (TP) and pine plantation (PP) as forest, emissions and removals in areas converted from the other forest categories to these two categories were excluded from the National FRL submitted in 2017 in consideration of safeguards, while those in areas converted from TP and PP to the other land use categories are included. The project participant uses the latest National FRL, and apply zero as EF for the conversion categories which are excluded from calculation in the National FRL. #### Option 1: Use the National FRL transition probabilities from forest to non-forest classes only EFi is an emission factor for area of land converted from land use category i, which is forest, to non-forest. EFi = kCik Equation 44 Where: EFi Emission factor applicable for forest class i; tC ha-1 Cik Carbon stock in carbon pool k in forest class i per unit area; tC ha-1 k Carbon pools included in establishment of National FRL; dimensionless Above ground and below ground are the carbon pools included in the establishment of the National FRL submitted in 2017. Table 2 shows the calculated EFi based on the National FRL submitted in 2017. Table 2 Emission factors, EFi, for Option 1 based on the National FRL submitted in 2017 Forest class, i; Abbreviation; EFi, tC ha-1. Evergreen forest; E; 91.30. Semi-evergreen forest; SE; 135.11. Pine forest; P; 56.54. Deciduous forest; D; 48.21. Bamboo; B; 0. Mangrove; M; 84.15. Rear Mangrove; MR; 92.40. Flooded forest; FF; 39.86. Forest regrowth; FR; 42.65. Tree plantation; TP; 56.54. Pine plantation; PP; 56.54. Non-forest; NF; 0. Option 2: Use all the National FRL transition probabilities among classes that would result in emissions EFij is an emission factor for area of land converted from land use category i to j. EFij = kCik- kCjk Equation 45 Where: EFij Emission factor for area of land converted from land use category i to j; tC ha-1 Cik Carbon stock in carbon pool k in land use category i per unit area; tC ha-1 k Carbon pools included in establishment of National FRL; dimensionless Table 3 Emission factors, EFij, for Option 2 based on the National FRL submitted in 2017 Land use category j. E; SE; P; D; B; M; MR; FF; FR; TP; PP; NF. i; E; 0.00; NA; 34.76; 43.09; 91.30; 7.15; NA; 51.44; 48.65; NA; NA; 91.30. SE; 43.81; 0.00; 78.57; 86.90; 135.11; 50.96; 42.71; 95.25; 92.46; NA; NA; 135.11. P; NA; NA; 0.00; 8.33; 56.54; NA; NA; 16.68; 13.89; NA; NA; 56.54. D; NA; NA; NA; 0.00; 48.21; NA; NA; 8.35; 5.56; NA; NA; 48.21. B; NA; NA; NA; NA; 0.00; NA; NA; NA; NA; NA; NA; 0.00. M; NA; NA; 27.61; 35.94; 84.15; 0.00; NA; 44.29; 41.50; NA; NA; 84.15. MR; 1.10; NA; 35.86; 44.19; 92.40; 8.25; 0.00; 52.54; 49.75; NA; NA; 92.40. FF; NA; NA; NA; NA; 39.86; NA; NA; 0.00; NA; NA; NA; 39.86. FR; NA; NA; NA; NA; 42.65; NA; NA; 2.79; 0.00; NA; NA; 42.65. TP; NA; NA; 0.00; 8.33; 56.54; NA; NA; 16.68; 13.89; 0.00; 0.00; 56.54. PP; NA; NA; 0.00; 8.33; 56.54; NA; NA; 16.68; 13.89; 0.00; 0.00; 56.54. NF; NA; NA; NA; NA; 0.00; NA; NA; NA; NA; NA; NA; 0.00. Note that when a land use category with a higher carbon stock is converted to a lower carbon stock, e.g., conversion from evergreen forest to non-forest, the EFij is positive. Removals are not included in this methodology, and therefore emission factors for conversions from a land use category with lower carbon stock to higher carbon stock are not available in Table 3. As previously described, emissions and removals in areas converted from the other forest categories to tree plantation (TP) or pine plantation (PP) were excluded in the establishment of the National FRL submitted in 2017, and therefore EFs of those transitions as well as transitions resulting in removals are also not available in Table 3. Area of land use category i at the inception of the project This method applies to both options, Option 1 and 2, and to the following parameters: Ai 0 Area of forest class i or land use category i in the project area at the inception of the project; ha Ad i 0 Area of forest class i or land use category i in the displacement belt at the inception of the project; ha The most recent version of Cambodia’s official forest map (≤ 2 years) provided by Cambodian government used to determine the area of land use category i in the project area and in the displacement belt at the project start date. Once Ai 0 and Ad i 0 are determined in the project design document, these areas, determined at the project start date, will not be updated. To quantify and account for the uncertainty of Cambodia’s official forest map in the project area, an accuracy assessment will be performed as described in Section H calculation of project net emission, sub-section (1) carbon stock change in the project area. Annual transition probability in the reference area Option 1: Use the National FRL transition probabilities from forest to non-forest classes only Annual deforestation rate for each forest type existing inside the project area, Pi, is calculated by applying area, Aref i, and area change, CAref i, from the National FRL and nationally endorsed data sets, and is used in Equation 2. Table 4 shows Pi calculated based on the National FRL submitted in 2017. Pi = CAref i / (Aref i * Tref) Equation 46 Where: Pi Annual transition probability from forest class i to non-forest within the reference area; dimensionless, 0-1 CAref i Area converted into non-forest from forest class i during the reference period, e.g., 2006-2014 in the National FRL submitted in 2017, in reference area; ha Aref i Area of forest class i in the reference area at the first year, e.g., 2006 in the National FRL submitted in 2017, of the reference period; ha Tref Number of years, e.g., eight in the National FRL submitted in 2017, in the reference period; year Table 4 Pi calculated based on the data used in the National FRL submitted in 2017 Forest class; Abbreviation; Pi, dimensionless. Evergreen forest; E; 0.0249. Semi-evergreen forest; SE; 0.0309. Pine forest; P; 0.0000. Deciduous forest; D; 0.0345. Bamboo; B; 0.0141. Mangrove; M; 0.0100. Rear Mangrove; MR; 0.0417. Flooded forest; FF; 0.0506. Forest regrowth; FR; 0.0972. Tree plantation; TP; 0.1169. Pine plantation; PP; 0.000. Option 2: Use all the National FRL transition probabilities among classes that would result in GHG emissions Annual transition probability from land use category i to j is obtained in the form of a transition probability matrix, MP, using the forest area change matrix reported in National FRL and nationally endorsed data sets. It is important to note that the annual transition probability cannot be obtained by simply dividing transition probability in multiple years by number of the years, because transition probability in multiple years, c years, is c-th power of annual transition probability. It is recommended to use a method provided in a peer reviewed scientific paper for calculation of the annual transition probability, such as Takada et al. 2010. Calculated MP is used in Equation 6. Obtain an area-based i-by-j transition matrix for each of time interval from the National FRL. In the National FRL submitted in 2017, Forest Area Change Matrices between 2006 and 2010 and between 2010 and 2014 area are available in Annex I Emission/Removal Calculation Tables. Annual transition probabilities derived using the matrices in the National FRL submitted in 2017 are available in Table 6 below. Transition probability in an observation interval t is calculated as follows: probij t = CAref ij t/Aref i t Equation 47 Where: probij t Transition probability from land use category i to j during an observation time interval t; dimensionless, 0-1 CAref ij t Area converted from land use category i to j during an observation time interval t within the reference area; ha Aref i t Area of land use category i at the first year of an observation time interval t within reference area; ha The notion probij t is an element of i-by-j cross-tabulation matrix MPmulti t, and MPmulti t is the Tmulti-th power of annual transition matrix MPt. MPmulti t= MPtTmulti Equation 48 Where: MPmulti t i-by-j matrix in which each element is the transition probability from land use category i to j during an observation time interval t, probij t; dimensionless, 0-1 MPt i-by-j matrix in which each element is the annual transition probability from land use category i to j during an observation time interval t, pij t; dimensionless, 0-1 Tmulti number of years of observation interval; year Table 5 shows the transition probability matrix, MPmulti 2006-2010, in which each element probij 2006-2010 is the transition probability from i to j in 4 years between 2006 and 2010 in the cross-tabulation based on the National FRL submitted in 2017. Note that the diagonal elements represent the probabilities of land being in the same category. Table 5 Cross-tabulation matrix of 4-year, 2006-2010, transition probability based on the National FRL submitted in 2017 2010. 2006; E; SE; P; D; B; M; MR; FF; FR; TP; PP; NF. E; 0.960; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.001; 0.000; 0.000; 0.039. Se; 0.000; 0.952; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.002; 0.000; 0.000; 0.045. P; 0.000; 0.000; 1.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000. D; 0.000; 0.000; 0.000; 0.971; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.029. B; 0.000; 0.000; 0.000; 0.000; 0.992; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.008. M; 0.004; 0.002; 0.000; 0.000; 0.000; 0.968; 0.001; 0.000; 0.000; 0.000; 0.000; 0.025. MR; 0.000; 0.003; 0.000; 0.000; 0.000; 0.003; 0.962; 0.000; 0.002; 0.000; 0.000; 0.030. FF; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.790; 0.001; 0.000; 0.000; 0.209. FR; 0.001; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.881; 0.001; 0.000; 0.116. TP; 0.000; 0.000; 0.000; 0.001; 0.000; 0.000; 0.000; 0.000; 0.004; 0.184; 0.000; 0.811. PP. NF; 0.002; 0.001; 0.000; 0.003; 0.000; 0.000; 0.000; 0.007; 0.007; 0.001; 0.000; 0.979. Obtain the annual transition probability matrix, MPt, in which each element pij t is the annual transition probability during an observation time interval t, by using MPmulti t and a method provided in a peer reviewed scientific paper for calculation of annual transition probability such as Takada et al. 2010. The obtained MPt should be validated by applying Equation 44. If multiple MPts from more than one observation time interval are derived, calculate average annual transition probability as follows: pij= tpij tNint Equation 49 Where: pij Annual transition probability from land use category i to j within the reference area; dimensionless, 0-1 pij t Annual transition probability from land use category i to jduring an observation time interval t; dimensionless, 0-1 Nint Number of observation intervals reported in National FRL; interval Table 6 shows the annual transition probability matrix, MP, in which each element pij is annual transition probability from i to j based on observations during intervals of 2006-2010 and 2010-2014 presented in the National FRL submitted in 2017. Table 6 Annual transition probability matrix based on observations during intervals of 2006-2010 and 2010-2014 presented in the National FRL submitted in 2017 y+1. y; E; SE; P; D; B; M; MR; FF; FR; TP; PP; NF. E; 0.971; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.002; 0.000; 0.000; 0.027. SE; 0.000; 0.963; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.002; 0.000; 0.000; 0.035. P; 0.000; 0.000; 1.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000. D; 0.000; 0.000; 0.000; 0.960; 0.000; 0.000; 0.000; 0.000; 0.000; 0.001; 0.000; 0.040. B; 0.000; 0.000; 0.000; 0.000; 0.985; 0.000; 0.000; 0.000; 0.001; 0.000; 0.000; 0.015. M; 0.001; 0.000; 0.000; 0.000; 0.000; 0.988; 0.001; 0.000; 0.000; 0.000; 0.000; 0.010. MR; 0.000; 0.000; 0.000; 0.000; 0.000; 0.003; 0.944; 0.000; 0.005; 0.001; 0.000; 0.048. FF; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.940; 0.001; 0.000; 0.000; 0.060. FR; 0.004; 0.002; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.875; 0.002; 0.000; 0.117. TP; 0.000; 0.000; 0.000; -0.001; 0.000; 0.000; 0.000; -0.001; 0.003; 0.780; 0.000; 0.219. PP; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 0.000; 1.000; 0.000. NF; 0.001; 0.001; 0.000; 0.003; 0.000; 0.000; 0.000; 0.002; 0.002; 0.000; 0.000; 0.990. Annual transition probability in the displacement belt The same method as (3), Annual transition probability in the reference area, is applied for the displacement belt instead of the reference area. Parameters for calculating emissions due to the project activities Fossil fuel combustion Default net caloric value and net calorific based emission factors are available in the 2006 IPCC Guidelines, and those of selected fuels types are listed in the table for “the source of each data and parameter fixed ex ante”. Reference figures such as manufacturer specifications can be used for SECj f. If no specific energy consumption data are available, fuel consumption and distance and/or hours are recorded before the initial verification to calculate SECj f. Those figures can be used for the entire project period. Fertilizer application Data and parameter fixed ex ante and default values for calculating N2O and CO2 emissions are shown in the tables for “the source of each data and parameter fixed ex ante” and “the source of each of default values selected in this methodology” below. The source of each data and parameter fixed ex ante is listed as below. Parameter; Description of data; Source. Ai 0; Area of forest class i or land use category i in the project area at the inception of the project; ha; Cambodia's official forest map. Pi; Option (1) Annual transition probability from forest class i to non-forest within the reference area; dimensionless, 0-1; Cambodia’s official forest reference level (National FRL). dy; Number of operating days in year y; Decided based on starting date of project operation and expected operational lifetime of project. do y; Number of days in year y; Automatically decided by number of year y. pij; Option (2) Annual transition probability from land use category i to j within the reference area; dimensionless, 0-1; Cambodia’s official forest reference level (National FRL). EFi; Option (1) Emission factor applicable for forest class i; tC ha-1; Cambodia’s official forest reference level (National FRL). EFij; Option (2) Emission factor for area of land converted from land use category i to j; tC ha-1; Cambodia’s official forest reference level (National FRL). Ad i 0; Area of forest class i in the displacement belt at the inception of the project; ha; Cambodia's official forest map. Pd i; Option (1) Annual transition probability from forest class i to non-forest within the displacement belt; dimensionless, 0-1; Cambodia’s official forest maps. pd ij; Option (2) Annual transition probability from land use category i to j within the displacement belt; dimensionless, 0-1; Cambodia’s official forest maps. NCVf; Net calorific value of fuel f; GJ kg-1 - Gas/Diesel oil: 43.0 TJ Gg-1 - Motor Gasoline: 44.3 TJ Gg-1 - Crude Oil: 42.3 TJ Gg-1; Table 1.2 of Ch. 1 Vol. 2 of 2006 IPCC Guidelines. EFfuel f; CO2 emission factor of the fuel type f combusted; t CO2 GJ-1 - Gas/Diesel Oil: 74,100 kg CO2 TJ-1 - Motor Gasoline: 69,300 kg CO2 TJ-1 - Crude Oil: 73,300 kg TJ-1; Table 3.2.1 of Ch. 3 and Table 2.5 of Ch.2, Vol. 2 of 2006 IPCC Guidelines Tables 2.5 and 3.2.1. The source of each of default values selected in this methodology is listed as below. Parameter; Description of data; Source. EFdirect-N c; Emission factor for N2O emissions from N inputs in cropland type c; tN2O-N (tN-input)-1 - Cropland in general: 0.01 tN2O-N (tN-input)-1 - Rice paddy (flooded rice field): 0.003 tN2O-N (tN-input)-1; Table 11.1 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. EFindirect-N; Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces; tN2O-N (t NH3-N and NOX-N volatilized)-1 - 0.010 t N2O-N (t NH3-N and NOX-N volatilized)-1; Table 11.3 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. EFleach-N; Emission factor for N2O emissions from N leaching and runoff; tN2O-N (t leaching and runoff)-1 - 0.0075 tN2O-N (t N leaching and runoff)-1; Table 11.3 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. EFlimestone; Emission factor for limestone; tC (t limestone)-1 - 0.12 tC (t limestone)-1; Section 11.3.1 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. EFdolomite; Emission factor for dolomite; tC (t dolomite)-1 - 0.13 tC (t dolomite)-1; Section 11.3.1 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. EFurea; Emission factor for urea; tC (t urea)-1 - 0.20 tC (t urea)-1; Section 11.4.1 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. GWPN2O; Global Warming Potential for N2O; tCO2 (tN2O)-1 - GWP100 for N2O: 298 tCO2 (tN2O)-1; Table 2.14 in Ch.2 of Working Group I contribution to the IPCC Forth Assessment Report. FracSN; Fraction that volatilized as NH3 and NOX for synthetic fertilizers; dimensionless, 0-1 - 0.10; Table 11.3 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. FracON; Fraction that volatilized as NH3 and NOX for organic fertilizers; dimensionless, 0-1 - 0.20; Table 11.3 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines. Fracleach; Fraction of N that area lost through leaching and runoff; dimensionless, 0-1 - 0.30; Table 11.3 of Ch. 11 Vol. 4 of 2006 IPCC Guidelines.

History of the document

| Version | Date | Contents revised |
| --- | --- | --- |
| 01.0 | 21 February 2020 | JC5, Annex 5 Initial approval. |
| 01.1 | 27 February 2021 | Electronic decision by the Joint Committee - Revised indication of years in the spreadsheet. - Added a parameter of “number of operating days in year y” and adjusted the equations including the parameter. |

A/R methodological tool “Estimation of GHG emissions related to fossil fuel combustion in A/R CDM project activities” was referenced.

This is based on A/R Methodology tool “Estimation of direct nitrous oxide emission from nitrogen fertilization” and 2006 IPCC Guidelines for National Greenhouse Gas Inventories.

Takada T, Miyamoto A and Hasegawa SF (2010) Derivation of a yearly transition probability matrix for land-use dynamics and its applications. Landscape Ecol 25:561-572