Joint Crediting Mechanism Approved Methodology VN_AM002

“Introduction of room air conditioners equipped with inverters”

- Title of the methodology

Introduction of room air conditioners equipped with inverters, Version 01.0

- Terms and definitions

| Terms | Definitions |
| --- | --- |
| Room air conditioner (RAC) | A single split type air conditioner. |
| Inverter | A device included in RACs and other motor-operated appliances, whose function is to vary the speed of the compressor motor in line with different load demand, for example to enable variable refrigerant flow to optimally regulate the temperature. |
| Public sector buildings | Buildings owned or administered by national or local government. |
| Energy efficiency ratio (EER) | The ratio of total cooling capacity to rated input power in specified conditions. |
| Cooling seasonal performance factor (CSPF) | Energy efficiency of RACs factoring into the seasonal temperature variation. Ratio of the total annual amount of heat that the RAC can remove from the indoor air when operated for cooling active mode to the total annual amount of energy consumed by the equipment during the same period. |

- Summary of the methodology

| Items | Summary |
| --- | --- |
| GHG emission reduction measures | Energy saving achieved by introduction of RACs equipped with inverters. |
| Calculation of reference emissions | GHG emissions associated with electricity consumption of reference RACs are calculated based on the monitored electricity consumption of project RACs, the ratio of the energy efficiency of reference and project RACs, and the CO2 emission factor of the electricity consumed by project RACs. |
| Calculation of project emissions | GHG emissions associated with electricity consumption of project RACs are calculated based on the monitored electricity consumption of project RACs and the CO2 emission factor of the electricity consumed by project RACs. |
| Monitoring parameters | Electricity consumption of project RACs Project energy efficiency (CSPF of project RACs) Reference energy efficiency (CSPF of reference RACs) |

- Eligibility criteria

This methodology is applicable to projects that satisfy all of the following criteria.

| Criterion 1 | The methodology is applicable to the following types of projects: - Installation of inverter RACs to public sector buildings. - Replacement of existing non-inverter RACs by inverter RACs in all types of buildings. |
| --- | --- |
| Criterion 2 | Rated cooling capacity of a project RAC is within the applicable range of the Vietnamese national standard TCVN7831:2012. |
| Criterion 3 | Ozone Depletion Potential (ODP) of the refrigerant used for project RAC is zero. |
| Criterion 4 | Plans to prevent release of refrigerants into the atmosphere at the time of RAC removal are prepared for both project RACs and the existing RACs replaced by the project. In the case of replacing existing RACs by project RACs, execution of the prevention plan is checked at the time of verification, in order to confirm that refrigerant used for the existing RACs removed by the project is not released to the air. |

- Emission Sources and GHG types

| Reference emissions |  |
| --- | --- |
| Emission sources | GHG types |
| Electricity consumption by reference RACs | CO2 |
| Project emissions |  |
| Emission sources | GHG types |
| Electricity consumption by project RACs | CO2 |

- Establishment and calculation of reference emissions

F.1. Establishment of reference emissions

Reference emissions are established as the product of monitored electricity consumption of project RACs, the ratio of the energy efficiency of reference and project RACs, and the CO2 emission factor of the electricity consumed by project RACs. The methodology provides following stepwise procedures to set energy efficiency values of the reference and project RACs, ex-post. In the procedures, reference RACs are conservatively set to results in a net reduction of emissions. Step 1: Determine reference RACs that lead to net emission reduction Select a reference RAC for each model of project RAC which meets the following conditions: - Not equipped with inverters. - Categorized as Grade 4 of the energy efficiency grades by EER as outlined in Table 3 of Vietnamese national standard TCVN7830:2012. - Cooling capacity of the reference RAC selected for the purpose of calculating reference emissions belongs to the same rated capacity class as the project RAC, based on the three rated capacity classes in Table 3 of TCVN7830:2012. - Reference RAC is previously unused and is currently available in the market at the time of CSPF determination. Step 2: Determine CSPF of reference RACs CSPF values of selected reference RACs by step 1 are determined at a third party testing facility which is equipped with a calorimeter capable of determining CSPF in line with ISO5151, following the testing procedures and conditions outlined in the latest version of Vietnamese National Standard TCVN 7831 at the time of CSPF determination. Step 3: Determine CSPF of project RACs CSPF values of each model type of project RACs are determined at a third party testing facility which is equipped with a calorimeter capable of determining CSPF in line with ISO5151, following the testing procedures and conditions outlined in the latest version of Vietnamese National Standard TCVN 7831 at the time of CSPF determination. Step 4: Select the reference and project energy efficiency (CSPF) values for the project Among the CSPF values calculated in Step 2 and 3, select the highest value of CSPF determined according to step 2 and the lowest value of CSPF determined according to step 3 to yield the efficiency ratio (PJ / ηREF in equation 1). These values are used as the reference and project CSPF values during the project lifetime. This step ensures that ratio of CSPF values used for the purpose of calculating reference emissions is conservatively derived for the project.

F.2. Calculation of reference emissions

(1) Where REp; =; Reference emissions during the period p [tCO2/p]. ECPJ,i,p; =; Electricity consumption by project RACs group i during the period p [MWh/p]. n; =; Number of RACs groups whose aggregate electricity consumption are measured by one electricity meter [dimensionless]. i; =; An index variable that is used to count the number of RACs groups. REF; =; Highest energy efficiency (CSPF) of reference RACs [dimensionless]. PJ; =; Lowest energy efficiency (CSPF) of project RACs [dimensionless]. EFelec; =; CO2 emission factor of electricity consumed [tCO2/MWh].

- Calculation of project emissions

(2) Where PEp; =; Project emissions during the period p [tCO2/p]. ECPJ,i,p; =; Electricity consumption by project RACs group i during the period p [MWh/p]. EFelec; =; CO2 emission factor of electricity consumed [tCO2/MWh].

- Calculation of emissions reductions

(3) Where ERp; =; Emission reductions during the period p [tCO2/p]. REp; =; Reference emissions during the period p [tCO2/p]. PEp; =; Project emissions during the period p [tCO2/p].

- Data and parameters fixed ex ante

| Parameter | Description of data | Source |
| --- | --- | --- |
| EFelec | CO2 emission factor of electricity consumed. When captive power generation is not available at the project site, then the most recent Vietnamese national grid emission factor [EFgrid] available at the time of validation is applied as [EFelec] and fixed for the monitoring period thereafter. When captive power generation is available at the project site, then [EFelec] is conservatively selected as below and fixed for the monitoring period thereafter: EFelec = min(EFgrid, EFcaptive) EFcaptive = 0.8 tCO2/MWh* *The most recent emission factor available from CDM approved small scale methodology AMS-I.A at the time of validation is applied. | [EFgrid] Ministry of Natural Resources and Environment of Vietnam (MONRE), Vietnamese DNA for CDM unless otherwise instructed by the Joint Committee. [EFcaptive] CDM approved small scale methodology: AMS-I.A |
| n | Number of RACs groups whose aggregate electricity consumption are measured by one electricity meter [dimensionless] | The project proponent selects an integer between 1 and 25 in line with the number of RACs groups included in the project. |

History of the document

| Version | Date | Contents revised |
| --- | --- | --- |
| 01.0 | 14 January 2015 | JC3, Annex 3 Initial approval. |

CSPF of the reference RAC selected using steps as stipulated in Section F.1.

CSPF of the project RAC selected using steps as stipulated in Section F.1.