Joint Crediting Mechanism Approved Methodology VN_AM004

“Anaerobic digestion of organic waste for biogas utilization within wholesale markets”

- Title of the methodology

- Anaerobic digestion of organic waste for biogas utilization within wholesale markets (Version 1.0)

- Terms and definitions

| Terms | Definitions |
| --- | --- |
| Air tightness test | A test to confirm that there is no leakage from the gas tanks and its connecting pipes once continuous anaerobic digesters have been installed. The test is conducted based on JIS B8266, ISO 16528-1 or ASME Boiler and Pressure Vessel Code. |
| Biogas | Gases generated from anaerobic digesters. |
| Continuous anaerobic digester | A biogas recovery system through continuous methane fermentation process from undiluted organic waste. The system consists of conditioning tanks and methane fermentation tanks, inside temperature of which are controlled to keep mesophilous condition. |
| Organic waste | Solid waste that contains degradable organic matter. This may include, for example, food waste, plant waste and sludge from wastewater treatment plants. |
| Periodical check | A periodical maintenance operation done by the manufacturer or an agent who is authorized by the manufacturer to maintain the waste management facility performance (not including partial replacement or overhaul). |
| Waste management facility | A set of facilities such as management buildings, waste receiving/storage areas, continuous anaerobic digesters, storage tanks and residuals treatment equipment (if any) located within a wholesale market to treat waste for biogas generation. |

- Summary of the methodology

| - Items | - Summary |
| --- | --- |
| - GHG emission reduction measures | - This methodology comprises measures to avoid the emissions of methane to the atmosphere from organic waste that have been left to decay anaerobically at a Solid Waste Disposal Site (SWDS) and to introduce renewable energy technologies that supply users with biogas that displaces fossil fuel use. |
| - Calculation of reference emissions | - For avoidance of methane emissions, the reference emissions are calculated based on the weight of organic waste prevented from disposal at the SWDS using first-order decay (FOD) model. - For renewable energy technologies that displace technologies using fossil fuel, the reference emissions are calculated based on the monitored amount of biogas supplied, Net Calorific Value (NCV) of the biogas and CO2 emission factor of the reference fossil fuel. |
| - Calculation of project emissions | - Project emissions are calculated on the basis of monitored electricity consumption. |
| - Monitoring parameters | - Amount of organic waste prevented from disposal in the SWDS excluding sludge - Amount of processed biogas supplied to heat generation equipments |

- Eligibility criteria

- This methodology is applicable to projects that satisfy all of the following criteria.

| Criterion 1 | The project installs continuous anaerobic digesters within a wholesale market whose organic wastes have been disposed at a SWDS where the generated landfill gas is not recovered, and utilizes generated biogas for heat generation. |
| --- | --- |
| Criterion 2 | The waste management facility to which the project continuous anaerobic digesters are installed is equipped with a device to remove hydrogen sulphide from the biogas before the generated gas is fed into the gas holders and tanks. |
| Criterion 3 | Air tightness test is conducted at least once before starting operation of the continuous anaerobic digesters. |
| Criterion 4 | Plan to avoid methane emissions from the residuals from project anaerobic digestion is prepared (e.g. including economic use such as sold as fertilizer or other appropriate treatment). |
| Criterion 5 | Periodical check at least once a year is planned. |

- Emission Sources and GHG types

| Reference emissions |  |
| --- | --- |
| Emission sources | GHG types |
| Methane emissions from decay of organic waste | CH4 |
| Fossil fuel consumption by the heat generation equipments | CO2 |
| Project emissions |  |
| Emission sources | GHG types |
| Grid electricity consumption by the waste management facility | CO2 |

- Establishment and calculation of reference emissions

F.1. Establishment of reference emissions

Reference emissions consist of two types of emission sources: - Methane emissions from decay of organic waste at SWDS - Fossil fuel consumption by the heat generation equipments 1) Calculation of reference emissions from decay of organic waste at SWDS Organic waste from wholesale markets is typically landfilled at SWDSs and anaerobically digested which leads to methane emissions to the atmosphere. The reference emissions from decay of organic waste are calculated using the FOD model adopted in the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. 2) Calculation of reference emissions from fossil fuel consumption by the heat generation equipments The generated biogas from anaerobic digesters replaces the fossil fuel which is used for heat generation by the heat generation equipments within the wholesale market. The reference emissions from fossil fuel consumption are calculated by multiplying the amount of biogas supplied to the heat generation equipments, the NCV of the biogas and CO2 emission factor of the reference fossil fuel. [Net emission reductions] - Net emission reductions are achieved in this methodology by setting the default DOC value conservatively in line with 2006 IPCC Guidelines for National Greenhouse Gas Inventories as the followings: - Food waste, which has the lowest DOC value among organic waste types, is assumed to represent the organic waste from wholesale markets - Default DOC value of 8%, which is the lower value of the range 8-20% for food waste, is applied

F.2. Calculation of reference emissions

REp=RECH4,p+REFF,p Where REp Reference emissions during the period p [tCO2/p] RECH4,p Reference emissions from decay of organic waste during the period p [tCO2/p] REFF, p Reference emissions from fossil fuel consumption for heat generation during the period p [tCO2/p] Reference emissions from decay of organic waste during the period p (RECH4,p) is accounted only after 13 months have passed from the first disposal at the SWDS due to delay in generation of CH4 from the time of disposal at the SWDS. RECH4,p=m=p_startp_end 1-f×GWPCH4×1-OX×1612×F×DOCf×MCF×x=1m-13Wx×DOC×e-k12(m-13-x)×1-e-k12 Where RECH4,p Reference emissions from decay of organic waste during the period p [tCO2/p] f Fraction of methane captured at the SWDS and flared, combusted or used in another manner that prevents the emissions of methane to the atmosphere GWPCH4 Global warming potential (GWP) of methane OX Oxidation factor (reflecting the amount of methane from SWDS that is oxidized in the soil or other material covering the waste) 16/12 Molecular weight ratio of methane and carbon F Fraction of methane in the SWDS gas [volume fraction] DOCf Fraction of degradable organic carbon (DOC) that decomposes under specific conditions occurring in the SWDS [weight fraction] MCF Methane correction factor Wx Amount of organic waste prevented from disposal in the SWDS in month x [t] DOC Fraction of degradable organic carbon (by weight) [weight fraction] k Decay rate [1/year] x Months in the time period in which waste is disposed at the SWDS, extending from the first month in the time period (x=1) to month m (x=m) m The Nth month from the first disposal at the SWDS, extending from the first month of the period p (m=p_start) to the last month of the period p (m=p_end) p_start The Nth month from the first disposal, which is the first month of the period p. If that month is smaller than 14 and p_end is larger than 13, p_start is set at 14 because CH4 generation can be accounted only after 13 months have passed since the first disposal at the SWDS. p_end The Nth month from the first disposal, which is the last month of the period p. If p_end is smaller than 14, CH4 generation cannot be accounted. REFF,p=REBG,p×NCVBG×EFCO2,i Where REFF,p Reference emissions from fossil fuel usage for heat generation during the period p [tCO2/p] REBG, p Amount of processed biogas supplied to heat generation equipments during the period p [t/p] NCVBG Net calorific value of the processed biogas [GJ/t] EFCO2,i CO2 emission factor of fossil fuel i [tCO2/GJ] - i Type of fossil fuel i consumed by the heat generation equipments

- Calculation of project emissions

PEp=PECp×EFelec Where PEp Project emissions during the period p [tCO2/p] PECp Amount of electricity consumption by the waste management facility during the period p [MWh/p] EFelec CO2 emission factor of the electricity consumed [tCO2/MWh]

- Calculation of emissions reductions

ERp=REp-PEp Where ERp GHG emission reductions during the period p [tCO2/p] REp Reference emissions during the period p [tCO2/p] PEp Project emissions during the period p [tCO2/p]

- Data and parameters fixed ex ante

The source of each data and parameter fixed ex ante is listed as below.

| Parameter | Description of data | Source |
| --- | --- | --- |
| EFelec | CO2 emissions factor of the electricity consumed [tCO2/MWh] When project waste management facility consumes only grid electricity or captive electricity, the project participant applies the CO2 emission factor respectively. When project waste management facility may consume both grid electricity and captive electricity, the project participant applies the CO2 emission factor with lower value. [CO2 emission factor] For grid electricity: The most recent value available from the source stated in this table at the time of validation For captive electricity: 0.8* [tCO2/MWh] *The most recent value available from CDM approved small scale methodology AMS-I.A at the time of validation is applied. | [Grid electricity] Ministry of Natural Resources and Environment (MONRE), Vietnamese DNA for CDM unless otherwise instructed by the Joint Committee. [Captive electricity] CDM approved small scale methodology AMS-I.A |
| EFCO2,i | CO2 emission factor of the fossil fuel i [tCO2/GJ] If the project supplies the biogas to the existing heat generation equipments, CO2 emission factor of the fossil fuel i which has been used in the existing equipments is applied. If the project supplies the biogas to new equipments, the CO2 emission factor of natural gas is applied. | Country specific data or IPCC default value from “2006 IPCC Guidelines for National Greenhouse Gas Inventories”. Lower limit value of the default CO2 emission factor is applied. |
| f | Fraction of methane captured at the SWDS and flared, combusted or used in another manner that prevents the emissions of methane to the atmosphere f=0 | Default value in the methodology |
| GWPCH4 | Global Warming Potential of methane (100-yr value) GWPCH4=25 | IPCC Fourth Assessment Report (2.10.2 Direct Global Warming Potentials, Table 2.14) |
| OX | Oxidation factor (reflecting the amount of methane from SWDS that is oxidized in the soil or other material covering the waste) Value of either 0.1 or 0 is applied to OX depending on the type of SWDS. Type of SWDS; OX default values. Managed1, unmanaged and uncategorised SWDS; 0. Managed covered with CH4 oxidising material2; 0.1. 1 Managed but not covered with aerated material 2 Examples: soil, compost | 2006 IPCC guidelines for National Greenhouse Gas Inventories (Volume 3, Table 3.2) |
| F | Fraction of methane in the SWDS gas [volume fraction] F=0.5 | 2006 IPCC guidelines for National Greenhouse Gas Inventories (Volume 5, Chapter 3, “FRACTION OF CH4 IN GENERATED LANDFILL GAS (F)”) |
| DOCf | Fraction of degradable organic carbon (DOC) that decomposes under specific conditions occurring in the SWDS [weight fraction] DOCf=0.5 | 2006 IPCC guidelines for National Greenhouse Gas Inventories (Volume 5, Table 2.4 and 2.5) |
| MCF | Methane correction factor Type of SWDS; Value. Anaerobic managed SWDS; 1.0. Semi-aerobic managed SWDS; 0.5. Unmanaged SWDS-deep; 0.8. Unmanaged-shallow SWDS or stockpiles that are considered SWDS; 0.4. In Ho Chi Minh City, Type of SWDSs is Anaerobic managed SWDS. | 2006 IPCC guidelines for National Greenhouse Gas Inventories (Volume 5, Table 3.1) |
| DOC | Fraction of degradable organic carbon (by weight) [weight fraction] DOC =0.08 Lower value of the range 8-20% for food waste set in IPCC 2006 Guidelines for National Greenhouse Gas Inventories is applied. | 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 5, Tables 2.4 and 2.5) |
| k | Decay rate [1/year] k=0.4 | 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 5, Table 3.3) |
| NCVBG | Net calorific value of the biogas [GJ/t] NCVBG = 50.4 | 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 2, Table 1.2) |

History of the document

| Version | Date | Contents revised |
| --- | --- | --- |
| 01.0 | 4 August 2015 | JC4, Annex 1 Initial approval. |